BOQ FINAL – PROVISIONING INFRASTRUCTURE

DTI Data Center – Lantai 3

- Baseline Assumption

| Item | Baseline |
| --- | --- |
| Phase | Phase-1 (Year-1) |
| Platform | FishOS – Private Cloud Provisioning |
| Tenant | 10–15 |
| VM target | ±150 VM |
| SLA | 99.9% |
| Rack | 1 rack (expandable) |
| Power IT load | ±18 kW (dengan buffer) |
| Network uplink | 1–2 Gbps |
| Public IP | /29 (opsi upgrade /28) |

- Logical Architecture

- Physical Architecture

- BOQ – Server

D1. Control Plane Cluster (HA)

| No | Item | Spesifikasi Ringkas | Qty |
| --- | --- | --- | --- |
| 1 | Control Node | 2U, 2×CPU 16 Core, RAM 128 GB | 3 |
| 2 | OS Disk | 2×480 GB SSD (RAID-1) | incl |
| 3 | Data Disk | NVMe 1–2 TB | incl |
| 4 | NIC | Dual 25 GbE | incl |

Catatan:

- 3 node = quorum & HA

- Isolated dari workload tenant → stabilitas provisioning

D2. Compute Pool (VM Workload)

| No | Item | Spesifikasi Ringkas | Qty |
| --- | --- | --- | --- |
| 1 | Compute Node | 2U, 2×CPU 32 Core | 6 |
| 2 | Memory | 256 GB RAM / node | incl |
| 3 | OS Disk | SSD | incl |
| 4 | NIC | Dual 25 GbE | incl |

Capacity note:

- ±384 core fisik

D3. Storage Pool

| No | Item | Spesifikasi Ringkas | Qty |
| --- | --- | --- | --- |
| 1 | Storage Node | 2U | 3 |
| 2 | Data Disk | 8×3.84 TB SSD / node | incl |
| 3 | OS Disk | 2×480 GB SSD (RAID-1) | incl |
| 4 | NIC | Dual 25 GbE | incl |

Capacity note:

- Raw ±92 TB

- Usable ±30 TB (replica 3)

- BOQ – NETWORK & SECURITY

E1. Data Center Fabric

| No | Item | Spesifikasi | Qty |
| --- | --- | --- | --- |
| 1 | TOR Switch | 25 GbE, redundant (A/B) | 2 |
| 2 | Uplink | 40/100 GbE ke core DC | 2 |
| 3 | OOB Switch | 1 GbE (iDRAC/iLO) | 1 |

Catatan:
25 GbE dipilih karena Ceph + east-west traffic (10 GbE berisiko bottleneck).

E2. Perimeter & Connectivity

| No | Item | Spesifikasi | Qty |
| --- | --- | --- | --- |
| 1 | Firewall | Enterprise FW, HA (Active-Standby) | 2 |
| 2 | VPN | IPSec / SSL VPN | incl |
| 3 | NAT / Floating IP | Via FW / platform | incl |

- BOQ – RACK, POWER & DC SUPPORT

F1. Rack & Power

| No | Item | Spesifikasi | Qty |
| --- | --- | --- | --- |
| 1 | Rack | 42U | 1 |
| 2 | Intelligent PDU | Dual A/B feed | 2 |
| 3 | Power Budget | Design 15–18 kW / rack | — |

Power context:

- Phase-1 IT load ±12 kW

- Dengan buffer → ±18 kW

F2. Cooling & Monitoring

| No | Item | Spesifikasi | Qty |
| --- | --- | --- | --- |
| 1 | Cooling | Existing DC (validated) | — |
| 2 | Env Monitoring | Temp / humidity / power | incl |

- BOQ – NETWORK BANDWIDTH & IP

G1. Internet / WAN

| Item | Baseline | Upgrade Option |
| --- | --- | --- |
| Dedicated Internet | 1 Gbps | 2×1 Gbps (dual ISP) |
| Latency target | Enterprise-grade | Same |
| SLA | ≥99.9% | ≥99.9% |

G2. IP Addressing

| Network Type | Allocation |
| --- | --- |
| Management Network | Private (RFC1918) |
| Storage Network | Private, isolated VLAN |
| Tenant Network | Private (large CIDR) |
| Public / Floating IP | /29 (5 usable) |
| Upgrade option | /28 (13 usable) |

Penggunaan Public IP (/29):

- Portal / landing page

- API endpoint

- VPN endpoint

- NAT gateway

- Reserve / future

- Summary

| Kategori | Jumlah |
| --- | --- |
| Total server | 12 unit |
| Rack | 1 rack |
| TOR switch | 2 unit |
| Firewall | 2 unit (HA) |
| Internet | 1–2 Gbps |
| Power design | ≤18 kW |
| Public IP | /29 (baseline) |