BOQ FINAL – PROVISIONING INFRASTRUCTURE

DTI Data Center – Lantai 3

- TECHNICAL SPECIFICATION

- Baseline Assumption

| Item | Baseline |
| --- | --- |
| Phase | Phase-1 (Year-1) |
| Platform | FishOS – Private Cloud Provisioning |
| Tenant | 10–15 |
| VM target | ±150 VM |
| SLA | 99.9% |
| Rack | 1 rack (expandable) |
| Power IT load | ±18 kW (dengan buffer) |
| Network uplink | 1–2 Gbps |
| Public IP | /29 (opsi upgrade /28) |

- Logical Architecture

- Physical Architecture

- BOQ – Server

D1. Control Plane Cluster (HA)

| No | Item | Spesifikasi Ringkas | Qty |
| --- | --- | --- | --- |
| 1 | Control Node | 2U, 2×CPU 16 Core, RAM 128 GB | 3 |
| 2 | OS Disk | 2×480 GB SSD (RAID-1) | incl |
| 3 | Data Disk | NVMe 1–2 TB | incl |
| 4 | NIC | Dual 25 GbE | incl |

Catatan:

- 3 node = quorum & HA

- Isolated dari workload tenant → stabilitas provisioning

D2. Compute Pool (VM Workload)

| No | Item | Spesifikasi Ringkas | Qty |
| --- | --- | --- | --- |
| 1 | Compute Node | 2U, 2×CPU 32 Core | 6 |
| 2 | Memory | 256 GB RAM / node | incl |
| 3 | OS Disk | SSD | incl |
| 4 | NIC | Dual 25 GbE | incl |

Capacity note:

- ±384 core fisik

D3. Storage Pool

| No | Item | Spesifikasi Ringkas | Qty |
| --- | --- | --- | --- |
| 1 | Storage Node | 2U | 3 |
| 2 | Data Disk | 8×3.84 TB SSD / node | incl |
| 3 | OS Disk | 2×480 GB SSD (RAID-1) | incl |
| 4 | NIC | Dual 25 GbE | incl |

Capacity note:

- Raw ±92 TB

- Usable ±30 TB (replica 3)

- BOQ – NETWORK & SECURITY

E1. Data Center Fabric

| No | Item | Spesifikasi | Qty |
| --- | --- | --- | --- |
| 1 | TOR Switch | 25 GbE, redundant (A/B) | 2 |
| 2 | Uplink | 40/100 GbE ke core DC | 2 |
| 3 | OOB Switch | 1 GbE (iDRAC/iLO) | 1 |

Catatan:
25 GbE dipilih karena Ceph + east-west traffic (10 GbE berisiko bottleneck).

E2. Perimeter & Connectivity

| No | Item | Spesifikasi | Qty |
| --- | --- | --- | --- |
| 1 | Firewall | Enterprise FW, HA (Active-Standby) | 2 |
| 2 | VPN | IPSec / SSL VPN | incl |
| 3 | NAT / Floating IP | Via FW / platform | incl |

- BOQ – RACK, POWER & DC SUPPORT

F1. Rack & Power

| No | Item | Spesifikasi | Qty |
| --- | --- | --- | --- |
| 1 | Rack | 42U | 1 |
| 2 | Intelligent PDU | Dual A/B feed | 2 |
| 3 | Power Budget | Design 15–18 kW / rack | — |

Power context:

- Phase-1 IT load ±12 kW

- Dengan buffer → ±18 kW

F2. Cooling & Monitoring

| No | Item | Spesifikasi | Qty |
| --- | --- | --- | --- |
| 1 | Cooling | Existing DC (validated) | — |
| 2 | Env Monitoring | Temp / humidity / power | incl |

- BOQ – NETWORK BANDWIDTH & IP

G1. Internet / WAN

| Item | Baseline | Upgrade Option |
| --- | --- | --- |
| Dedicated Internet | 1 Gbps | 2×1 Gbps (dual ISP) |
| Latency target | Enterprise-grade | Same |
| SLA | ≥99.9% | ≥99.9% |

G2. IP Addressing

| Network Type | Allocation |
| --- | --- |
| Management Network | Private (RFC1918) |
| Storage Network | Private, isolated VLAN |
| Tenant Network | Private (large CIDR) |
| Public / Floating IP | /29 (5 usable) |
| Upgrade option | /28 (13 usable) |

Penggunaan Public IP (/29):

- Portal / landing page

- API endpoint

- VPN endpoint

- NAT gateway

- Reserve / future

- Summary

| Kategori | Jumlah |
| --- | --- |
| Total server | 12 unit |
| Rack | 1 rack |
| TOR switch | 2 unit |
| Firewall | 2 unit (HA) |
| Internet | 1–2 Gbps |
| Power design | ≤18 kW |
| Public IP | /29 (baseline) |

- COST STRUCTURE

  - Cost Category

| No. | Category | Type |
| --- | --- | --- |
| 1 | Hardware Infrastructure | CAPEX |
| 2 | Software / Platform License | CAPEX / OPEX |
| 3 | Implementation (SI) | CAPEX |
| 4 | Hardware Maintenance | OPEX |
| 5 | Platform Support | OPEX |
| 6 | Power & Cooling | OPEX |
| 7 | Network & IP | OPEX |

  - CAPEX (One-Time)

    - Hardware Infrastructure  (Server and Storage)

      - Server – Control Plane (3 Unit)

Spesifikasi ringkas

- 2U server

- 2× CPU 16 core (Xeon / EPYC class)

- RAM 128 GB

- OS SSD RAID-1 + NVMe

- Dual PSU, OOB, Dual 25GbE NIC

Estimasi harga/unit

| Komponen | Estimasi (IDR) |
| --- | --- |
| Base server chassis + mobo | 120 jt |
| CPU (2× mid-range) | 90 jt |
| RAM 128 GB | 35 jt |
| SSD + NVMe | 25 jt |
| NIC 25GbE + OOB | 20 jt |
| PSU, rail kit, misc | 10 jt |
| Total / unit | 300 jt |

Subtotal  3 × 300 jt = 900 jt

      - Server – Compute Node (6 Unit)

Spesifikasi

- 2U

- 2× CPU 32 core

- RAM 256 GB

- OS SSD

- Dual 25GbE

Estimasi harga/unit

| Komponen | Estimasi |
| --- | --- |
| Base server | 130 jt |
| CPU (2× high core) | 140 jt |
| RAM 256 GB | 65 jt |
| OS SSD | 15 jt |
| NIC 25GbE | 20 jt |
| PSU, rail, misc | 10 jt |
| Total / unit | 380 jt |

Subtotal  6 × 380 jt = 2.280 jt

      - Server – Storage Node (3 Unit)

Spesifikasi

- 2U

- 8× 3.84 TB SSD (enterprise)

- OS SSD RAID-1

- Dual 25GbE

Estimasi harga/unit

| Komponen | Estimasi |
| --- | --- |
| Base storage server | 140 jt |
| SSD data (8×3.84 TB) | 160 jt |
| OS SSD | 15 jt |
| NIC 25GbE | 20 jt |
| RAID / HBA | 15 jt |
| PSU, rail, misc | 10 jt |
| Total / unit | 360 jt |

Subtotal  3 × 360 jt = 1.080 jt

    - Network – TOR & OOB Switch (Network, firewall, rack, PDU)

2.3.2.1TOR Switch 25GbE (2 unit)

- 25GbE enterprise switch

- ≥24 port

- Redundant A/B

| Item | Estimasi |
| --- | --- |
| TOR Switch 25GbE | 150 jt / unit |
| Optics / DAC | 40 jt / unit |
| Subtotal | 380 jt |

2.3.2.2 OOB Management Switch (1 unit)

| Item | Estimasi |
| --- | --- |
| 1GbE switch + cable | 30 jt |

2.3.2.3 Security – Firewall HA (2 UNIT)

Enterprise firewall

- HA active-standby

- IPS/VPN ready

| Item | Estimasi |
| --- | --- |
| Firewall appliance | 120 jt / unit |
| License year-1 | 30 jt / unit |
| Subtotal |  |

2.3.2.4 Rack & Power

| Item | Estimasi |
| --- | --- |
| Rack 42U | 50 jt |
| Intelligent PDU A/B | 25 jt × 2 = 50 jt |
| Cabling & accessories | 30 jt |
| Subtotal | 130 jt |

GRAND TOTAL HARDWARE CAPEX

| Category | IDR (juta) |
| --- | --- |
| Server (12 unit) | 4.260 |
| TOR + OOB Switch | 410 |
| Firewall HA | 300 |
| Rack & PDU | 130 |
| TOTAL | 5.100 |

  - Software / Platform License (Fish OS)

- Setup / initial license

- One-time enablement

👉 Asumsi: IDR 500 juta

  - Implementation

- Install & deploy FishOS

- Integration infra

- HA, security hardening

- UAT & KT

👉 Asumsi: IDR 500 juta

TOTAL CAPEX

Hardware               : 5.100.000.000

Platform setup         :   500.000.000

SI implementation      :   500.000.000

------------------------------------

TOTAL CAPEX            : 6.100.000.000 IDR

  - OPEX (Annual)

2.5.1 Hardware Maintenance

Asumsi: 10% × 5.0 M = 500 juta / tahun

2.5.2 Platform Support / License

- Support

- Patch & upgrade

- Escalation

Asumsi: 300 juta / tahun

2.5.3 Power & Cooling

- IT load ±18 kW

- 24×7

- DC internal charge

Asumsi:

18 kW × 24 × 365 ≈ 157.680 kWh

× IDR 1.600 ≈ 252 juta / tahun

(round up)
250 juta / tahun

2.5.4 Network & IP

- Internet dedicated 1–2 Gbps

- IP public (/29)

Asumsi:

- Internet: 200–300 juta / tahun

- IP & misc: 50 juta / tahun

350 juta / tahun

TOTAL OPEX / TAHUN

Hardware maintenance   : 500.000.000

Platform support       : 300.000.000

Power & cooling        : 250.000.000

Network & IP           : 350.000.000

------------------------------------

TOTAL OPEX / YEAR      : 1.400.000.000 IDR

2.6 TCO 3 TAHUN

CAPEX                  : 6.100.000.000

OPEX 3 tahun           : 1.400.000.000 × 3

= 4.200.000.000

------------------------------------

TOTAL TCO 3 TAHUN      : 10.300.000.000 IDR

- Breakdown Price – Business Model

3.1 Target Recovery

- Break-even ≤ 3 tahun

- Margin minimal 20–30%

Revenue target 3 tahun ≈ 13.5 – 14 M

3.2 Revenue per Tahun (Target)

14.000.000.000 / 3 ≈ 4.700.000.000 / tahun

3.3 Revenue per Bulan

≈ 390.000.000 / bulan

3.4 Turunkan ke Pricing Service

a. Model 1 – VM-based

Asumsi:

- 150 VM

- Utilisasi efektif 70% → 105 VM billable

390 juta / 105 VM ≈ 3.7 juta / VM / bulan

👉 Bisa dibulatkan jadi:

- Small VM: 2.0 – 2.5 jt

- Medium VM: 3.5 – 4.0 jt

- Large VM: 6.0 – 7.0 jt

b. Model 2 – Tenant-based (Enterprise friendly)

Asumsi:

- 12 tenant aktif

390 juta / 12 ≈ 32.5 juta / tenant / bulan

👉 Target segment customer:

- Enterprise

- Regulated customer

- Contract-based

c. Model 3 – Hybrid

- Base fee per tenant (platform)

  - usage VM

Contoh:

- Platform fee: 10–15 jt / tenant / bulan

- VM usage: sesuai size

“Dengan investasi ±6.2 M di awal dan OPEX ±1.5 M per tahun, platform provisioning ini membutuhkan revenue sekitar 4.5–5 M per tahun untuk break-even 3 tahun dengan margin sehat. Model pricing hybrid tenant + usage memberikan fleksibilitas dan menjaga utilisasi aset.”